# Jet and ring-sum algorithm technical documentation

Andrew W. Rose

# Abstract

Presented within this document are the specifications and implementation details for the components of the Jet and Ring-sum algorithms as currently implemented for the TDR calorimeter trigger upgrade.

Contents

Abstract	1

Design rules	5

Overview	7

Data types: Towers	9

tTower record	9

Array types	9

Data types: Jets	10

tJet record	10

tComparison record	10

tJetMaxima record	10

tFilteredJet record	10

Array types	10

Data types: Ring sums	12

tRingSegment record	12

tMHTcoefficients record	12

Array types	12

Algorithm: Tower Forming	13

Files	13

Generics	13

I/O	13

Latency	13

Description	14

Algorithm: 1×9 Strip Forming	15

Files	15

Generics	15

I/O	15

Latency	15

Description	15

Algorithm: 9×1 Strip Forming	17

Files	17

Generics	17

I/O	17

Latency	17

Description	17

Algorithm: 9×1 Vector Strip Forming	19

Files	19

Generics	19

I/O	19

Latency	19

Description	20

Notes	20

Algorithm: 9×9 Jet Sum	21

Files	21

Generics	21

I/O	21

Latency	21

Description	21

Notes	22

Algorithm: 9×9 Jet Veto	23

Files	23

Generics	23

I/O	23

Latency	23

Description	24

Notes	25

Algorithm: Donut pile-up estimation (DEPRECATED)	27

Files	27

Generics	27

I/O	27

Latency	28

Description	28

Notes	28

Algorithm: Donut pile-up estimation	29

Files	29

Generics	29

I/O	30

Latency	30

Description	30

Notes	30

Algorithm: Jet Overlap Filter	33

Files	33

Generics	33

I/O	33

Latency	34

Description	34

Notes	34

Algorithm: Pile-up subtraction	35

Files	35

Generics	35

I/O	35

Latency	36

Description	36

Notes	36

Algorithm: MHT coefficients	37

Files	37

Generics	37

I/O	37

Latency	37

Description	38

Algorithm: HT/MHT sums	39

Files	39

Generics	39

I/O	39

Latency	40

Description	40

Algorithm: ET/MET sums	41

Files	41

Generics	41

I/O	41

Latency	42

Description	42

Algorithm: HT/MHT and ET/MET accumulation	43

Files	43

Generics	43

I/O	43

Latency	43

Description	44

Notes	44

Algorithm: Jet Sorting	45

Files	45

Generics	45

I/O	45

Latency	45

Description	46

Notes	46

Algorithm: Accumulating Jet Sorting	47

Files	47

Generics	47

I/O	47

Latency	47

Description	48

Notes	48

Algorithm: Coordinate conversion	49

Files	49

Generics	49

I/O	49

Latency	49

Description	50

Notes	50

# Design rules

The following strategy is preferred and used in the Jet and Ring-Sum algorithm firmware:

- At top level, there shall be no clocked logic, only instances of entities

- All data will always be accompanied by a local data-valid flag

- Global counters, resets or data-valid flags are forbidden

- All signals connecting two entities shall be “pipes”: that is, data shall be placed into a clocked pipeline before leaving an entity

- Data will be clocked along the pipeline in order of increasing cell index

- Data will be clocked out of any logic and will not be clocked into the zeroth cell of the pipeline

- The eta-0 boundary will be handled using the muliplex-on-input principle

- CamelCase will be used throughout the firmware

- BLOCK CAPITALS will be used for all VHDL keywords

- Types will be prefixed with a lower-case “t”

- Constants will be prefixed with a lower-case “c”

- All entity port names will be appended with “In” or “Out” to immediately identify the direction of data into or out of the algorithm

# Overview

The Jet and Ring-sum algorithm and data flow is structured as shown in Figure 1.

Figure 1: Algorithm- and data-flow of the Jet and Ring-Sum algorithms.

# Data types: Towers

## tTower record

| EcalFG | STD_LOGIC | The Ecal finegrain flag |
| --- | --- | --- |
| HcalFG | STD_LOGIC | An Hcal feature bit |
| Energy | UNSIGNED( 8 DOWNTO 0 ) | The total E+H energy |
| Ecal | UNSIGNED( 16 DOWNTO 0 ) | An estimate of the Ecal energy |
| Hcal | UNSIGNED( 16 DOWNTO 0 ) | An estimate of the Hcal energy |
| DataValid | BOOLEAN | The data-valid flag propagated with the data |

## Array types

| tTowersInPhi | ARRAY( cTowersInPhi-1 DOWNTO 0 ) OF tTower |
| --- | --- |
| tTowersInEtaPhi | ARRAY( cRegionsInEta-1 DOWNTO 0 ) OF tTowersInPhi |
| tTowersPipe | ARRAY( NATURAL RANGE <> ) OF tTowersInEtaPhi |

- Towers are formed at every site: Number of towers in phi and number of regions are fixed by geometry of the detector

- The length of the pipe is determined by final application, so set as an unbounded array and let the length be assigned at usage

# Data types: Jets

## tJet record

| Energy | UNSIGNED( 15 DOWNTO 0 ) | The energy of the 9x9 jet or part thereof |
| --- | --- | --- |
| DataValid | BOOLEAN | The data-valid flag propagated with the data |

## tComparison record

| Data | BOOLEAN | The result of a comparison or of a Boolean-logic expression |
| --- | --- | --- |
| DataValid | BOOLEAN | The data-valid flag propagated with the data |

## tJetMaxima record

| Max | UNSIGNED( 8 DOWNTO 0 ) | The energy of the maximum-energy tower in a region |
| --- | --- | --- |
| Phi | INTEGER RANGE 0 TO 8 | The local phi-coordinate of the maximum-energy tower in a region |
| Eta | INTEGER RANGE 0 TO 3 | The local eta-coordinate of the maximum-energy tower in a region |
| DataValid | BOOLEAN | The data-valid flag propagated with the data |

## tFilteredJet record

| Energy | UNSIGNED( 15 DOWNTO 0 ) | The energy of the 9x9 jet or part thereof |
| --- | --- | --- |
| Phi | INTEGER RANGE 0 TO 71 | The global phi-coordinate of the jet |
| Eta | INTEGER RANGE 0 TO 35 | The global eta-coordinate of the jet |
| DataValid | BOOLEAN | The data-valid flag propagated with the data |

## Array types

| tJetsInPhi | ARRAY( cTowersInPhi-1 DOWNTO 0 ) OF tJet |
| --- | --- |
| tJetsInEtaPhi | ARRAY( cRegionsInEta-1 DOWNTO 0 ) OF tJetsInPhi |
| tJetsPipe | ARRAY( NATURAL RANGE <> ) OF tJetsInEtaPhi |

- Jets are formed at every site: Number of towers in phi and number of regions are fixed by geometry of the detector

- The length of the pipe is determined by final application, so set as an unbounded array and let the length be assigned at usage

| tComparisonsInPhi | ARRAY( cTowersInPhi-1 DOWNTO 0 ) OF tComparison |
| --- | --- |
| tComparisonsInEtaPhi | ARRAY( cRegionsInEta-1 DOWNTO 0 ) OF tComparisonsInPhi |
| tComparisonsPipe | ARRAY( NATURAL RANGE <> ) OF tComparisonsInEtaPhi |

- Jet veto calculation is performed at every site: Number of towers in phi and number of regions are fixed by geometry of the detector

- The length of the pipe is determined by final application, so set as an unbounded array and let the length be assigned at usage

| tJetMaximaInPhi | ARRAY( cTowersInPhi-1 DOWNTO 0 ) OF tJetMaxima |
| --- | --- |
| tJetMaximaInEtaPhi | ARRAY( cRegionsInEta-1 DOWNTO 0 ) OF tJetMaximaInPhi |
| tJetMaximaPipe | ARRAY( NATURAL RANGE <> ) OF tJetMaximaInEtaPhi |

- Jet maxima-finding calculation is performed within the Jet veto calculation at every site: Number of towers in phi and number of regions are fixed by geometry of the detector

- The length of the pipe is determined by final application, so set as an unbounded array and let the length be assigned at usage

| tFilteredJetsInPhi | ARRAY( NATURAL RANGE <> ) OF tFilteredJet |
| --- | --- |
| tFilteredJetsInEtaPhi | ARRAY( cRegionsInEta-1 DOWNTO 0 ) OF tFilteredJetsInPhi( ( cTowersInPhi / 4 ) -1 DOWNTO 0 ) |
| tFilteredJetsPipe | ARRAY( NATURAL RANGE <> ) OF tFilteredJetsInEtaPhi |

- The number of filtered jets in phi is determined by final application, so set as an unbounded array and let the length be assigned at usage.

- The main use case for the Filtered-Jet type is in handling the overlap-filtered jets. In this case, the number of jets in phi and number of regions are fixed by geometry of the detector and by the size of the jet

- The length of the pipe is determined by final application, so set as an unbounded array and let the length be assigned at usage

# Data types: Ring sums

## tRingSegment record

| Energy | UNSIGNED( 19 DOWNTO 0 ) | The total energy of all the towers or jets in a ring around phi or part thereof |
| --- | --- | --- |
| xComponent | SIGNED( 19 DOWNTO 0 ) | The x-component of the energy of all the towers or jets in a ring around phi or part thereof |
| yComponent | SIGNED( 19 DOWNTO 0 ) | The y-component of the energy of all the towers or jets in a ring around phi or part thereof |
| towersOverThreshold | UNSIGNED( 11 DOWNTO 0 ) | A count of the number of towers over threshold in a ring around phi or part thereof |
| DataValid | BOOLEAN | The data-valid flag propagated with the data |

## tMHTcoefficients record

| CosineCoefficients | SIGNED( 9 DOWNTO 0 ) | The value cosine(phi) with range -511 to 511 stored as a 10-bit signed integer |
| --- | --- | --- |
| SineCoefficients | SIGNED( 9 DOWNTO 0 ) | The value sine(phi) with range -511 to 511 stored as a 10-bit signed integer |

## Array types

| tRingSegmentsInPhi | ARRAY( cTowersInPhi-1 DOWNTO 0 ) OF tRingSegment |
| --- | --- |
| tRingSegmentsInEtaPhi | ARRAY( cRegionsInEta-1 DOWNTO 0 ) OF tRingSegmentsInPhi |
| tRingSegmentsPipe | ARRAY( NATURAL RANGE <> ) OF tRingSegmentsInEtaPhi |

- Ring Segments are formed at every site: Number of towers in phi and number of regions are fixed by geometry of the detector

- The length of the pipe is determined by final application, so set as an unbounded array and let the length be assigned at usage

| tRingSegmentsInEta | ARRAY( cRegionsInEta-1 DOWNTO 0 ) OF tRingSegment |
| --- | --- |
| tRingSegmentsPipe2 | ARRAY( NATURAL RANGE <> ) OF tRingSegmentsInEta |

- Once a ring is fully formed, there is no phi information, so drop it: number of regions are fixed by geometry of the detector

- The length of the pipe is determined by final application, so set as an unbounded array and let the length be assigned at usage

| tMHTcoefficientsInPhi | ARRAY( ( cTowersInPhi / 4 ) -1 DOWNTO 0 ) OF tMHTcoefficients |
| --- | --- |
| tMHTcoefficientsInEtaPhi | ARRAY( cRegionsInEta-1 DOWNTO 0 ) OF tMHTcoefficientsInPhi |
| tMHTcoefficientsPipe | ARRAY( NATURAL RANGE <> ) OF tMHTcoefficientsInEtaPhi |

- MHT coefficients are required for every filtered Jet: Number of towers in phi and number of regions are fixed by geometry of the detector and by the size of the jet

- The length of the pipe is determined by final application, so set as an unbounded array and let the length be assigned at usage

# Algorithm: Tower Forming

## Files

| Top-level | links/LinksIn.vhd |
| --- | --- |
| Types | components/mp7_datapath/firmware/hdl/mp7_data_types.vhd projects/calol2/algorithm_components/firmware/hdl/towers/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/towers/TowerPipe.vhd |
| Internal entities | projects/calol2/algorithm_components/firmware/hdl/towers/TowerFormer.vhd projects/calol2/algorithm_components/firmware/hdl/towers/TowerSum.vhd projects/calol2/algorithm_components/firmware/cgn/multiplier9Ux7U.xco |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/towers/functions_pkg.vhd |

## Generics

| Pipe lengths | towerPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | - |

## I/O

| Inputs | Name | linksIn |
| --- | --- | --- |
|  | Type | ldata( ( cNumberOfLinksIn-1 ) DOWNTO 0 ) |
|  | Description | Raw link data in. Note that this is “flat” data, not a pipe |
| Outputs | Name | towerPipeOut |
|  | Type | tTowersPipe( ( towerPipeOutLength-1 ) DOWNTO 0 ) |
|  | Description | Formatted towers, organized by eta-half/phi |

## Latency

| Clock cycles | 2 |
| --- | --- |
| BX | 13 |

## Description

- GENERATE loop over all links and for each link,

  - Split word into two 16-bit words

  - Pass each 16-bit word to TowerFormer

  - Assign output of TowerFormers to individual towers

- The TowerFormer entity then

  - Extracts energy and feature bits from 16-bit word

  - Estimates the Ecal and Hcal components of the tower based on the total energy and the E-over-H ratio

# Algorithm: 1×9 Strip Forming

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/Strip1x9Former.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/JetPipe.vhd |
| Internal entities | projects/calol2/algorithm_components/firmware/hdl/jets/JetSum.vhd |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd |

## Generics

| Pipe lengths | towerPipeInLength stripPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | - |

## I/O

| Inputs | Name | towerPipeIn |
| --- | --- | --- |
|  | Type | tTowersPipe( towerPipeInLength-1 DOWNTO 0 ) |
|  | Description | Formatted towers organized by eta-half/phi |
| Outputs | Name | strips1x9SumPipeOut |
|  | Type | tJetsPipe( stripPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Strips 1 in phi by 9 in eta, organized by eta-half/phi |

## Latency

| Clock cycles | 6 |
| --- | --- |
| BX | 1 |

## Description

- GENERATE loop over all towers in phi and both halves in eta and for each site

  - Sum three neighbouring towers in eta into a strip 1×3

  - Sum three non-overlapping neighbouring strip 1×3 objects in eta into a strip 1×9

# Algorithm: 9×1 Strip Forming

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/Strip9x1Former.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/JetPipe.vhd |
| Internal entities | projects/calol2/algorithm_components/firmware/hdl/jets/JetSum.vhd |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd |

## Generics

| Pipe lengths | towerPipeInLength stripPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | - |

## I/O

| Inputs | Name | towerPipeIn |
| --- | --- | --- |
|  | Type | tTowersPipe( towerPipeInLength-1 DOWNTO 0 ) |
|  | Description | Formatted towers organized by eta-half/phi |
| Outputs | Name | strips9x1SumPipeOut |
|  | Type | tJetsPipe( stripPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Strips 9 in phi by 1 in eta, organized by eta-half/phi |

## Latency

| Clock cycles | 2 |
| --- | --- |
| BX | 1/3 |

## Description

- GENERATE loop over all towers in phi and both halves in eta and for each site

  - Sum three neighbouring towers in phi into a strip 3×1

  - Sum three non-overlapping neighbouring strip 3×1 objects in eta into a strip 9×1

# Algorithm: 9×1 Vector Strip Forming

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/rings/VectorStrip9x1Former.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/rings/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/rings/RingPipe.vhd |
| Internal entities | projects/calol2/algorithm_components/firmware/cgn/multiplier9Ux10S.xco projects/calol2/algorithm_components/firmware/hdl/rings/RingSegmentSum.vhd |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/rings/functions_pkg.vhd |

## Generics

| Pipe lengths | towerPipeInLength stripPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | vectorTowerShift |

## I/O

| Inputs | Name | towerPipeIn |
| --- | --- | --- |
|  | Type | tTowersPipe( towerPipeInLength-1 DOWNTO 0 ) |
|  | Description | Formatted towers organized by eta-half/phi |
|  | Name | energyThresholdIn |
|  | Type | UNSIGNED( 10 DOWNTO 0 ) |
|  | Description | Energy threshold which will be used in the Towers-over-Threshold calculation |
| Outputs | Name | strips9x1SumPipeOut |
|  | Type | tRingSegmentsPipe( stripPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Ring-segments 9 in phi by 1 in eta, organized by eta-half/phi |

## Latency

| Clock cycles | 4 |
| --- | --- |
| BX | 2/3 |

## Description

- GENERATE loop over all towers in phi and both halves in eta and for each site

  - Compile-time calculate Sin(phi) and Cos(phi) from GENERATE parameter

  - Multiply energy by Sin(phi) and Cos(phi) to get x- and y-components of the vector sum

  - Right-shift the x- and y-components of the vector sum by vectorTowerShift bits and truncate the least-significant bits

  - Compare the tower energy to the input “energyThresholdIn” and, if greater than or equal to the threshold, label the tower as being over threshold.

- GENERATE loop over 1/3 towers in phi and both halves in eta and for each site

  - Sum three neighbouring towers in phi into a strip 3×1

- GENERATE loop over 1/9 towers in phi and both halves in eta and for each site

  - Sum three neighbouring strip 3×1 objects in eta into a strip 9×1

## Notes

Unlike the 1×9 Strip Forming and 9×1 Strip Forming, the 9×1 Vector Strip Forming does not create 72 overlapping 9-tower strips, but 8 non-overlapping 9-tower strips.

# Algorithm: 9×9 Jet Sum

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/JetFormer.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/JetPipe.vhd |
| Internal entities | projects/calol2/algorithm_components/firmware/hdl/jets/JetSum.vhd |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd |

## Generics

| Pipe lengths | stripPipeInLength jetPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | jetFormingOffset |

## I/O

| Inputs | Name | strip9x1PipeIn |
| --- | --- | --- |
|  | Type | tJetsPipe( stripPipeInLength-1 DOWNTO 0 ) |
|  | Description | Strips 9 in phi by 1 in eta, organized by eta-half/phi |
| Outputs | Name | jet9x9PipeOut |
|  | Type | tJetsPipe( jetPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Energy sums 9 in phi by 9 in eta, organized by eta-half/phi |

## Latency

| Clock cycles | 6 |
| --- | --- |
| BX | 1 |

## Description

- GENERATE loop over all towers in phi and both halves in eta and for each site

  - Sum three neighbouring 9×1 strips in eta into a 9×3 object

  - Sum three non-overlapping neighbouring 9×3 objects in eta into a 9×9 jet

## Notes

Because the pipes for the 9x1 strips are long anyway because of the donut pile-up subtraction and because the 9x9 sum is not needed until the jet veto is calculated, I have added a generic “jetFormingOffset” which delays the forming of the 9x9 sum until it is needed so that both the inputs (9x1 strips) and outputs (9x9 strips) are being stored in pipes simultaneously, thereby unnecessarily using resources.

# Algorithm: 9×9 Jet Veto

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/JetOverlapCriteria.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/JetPipe.vhd projects/calol2/algorithm_components/firmware/hdl/jets/JetComparisonPipe.vhd projects/calol2/algorithm_components/firmware/hdl/jets/JetMaximaPipe.vhd |
| Internal entities | - |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd |

## Generics

| Pipe lengths | towerPipeInLength overlapVetoPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | - |

## I/O

| Inputs | Name | towerPipeIn |
| --- | --- | --- |
|  | Type | tTowersPipe( towerPipeInLength-1 DOWNTO 0 ) |
|  | Description | Formatted towers organized by eta-half/phi |
| Outputs | Name | overlapVetoPipeOut |
|  | Type | tComparisonsPipe( overlapVetoPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Boolean specifying whether the central tower of the region 9 in phi by 9 in eta has been vetoed by another tower in the region, organized by eta-half/phi |

## Latency

| Clock cycles | 11 |
| --- | --- |
| BX | 1 5/6 |

## Description

- GENERATE loop over all towers in phi and both halves in eta and for each site

  - For each group of 3 neighbouring towers in phi, compare the towers and find the maximum energy.

  - Create a 3-tower maxima object which stores the maximum tower energy and the relative position of the maximum tower in phi (0-2)

  - For each group of 3 non-overlapping neighbouring 3-tower maxima objects in phi, compare the energy of each and find the maximum energy.

  - Create a 9-tower maxima object which stores the maximum tower energy and the relative position of the maximum tower in phi (0-8)

  - For each group of 3 neighbouring 9-tower maxima objects in eta, identify the maxima object satisfying the most stringent test criteria (as specified in Figure 2 and Figure 3). Because of the complexity of the logic here:

    - The comparisons of energies and phi are performed in one clock step

    - The combinatorial logic is performed in a second clock step setting a variable

    - The variable then selects the appropriate value for assignments to a 9×3-tower maxima object which stores the maximum tower energy, the relative position of the maximum tower in phi (0-8) and the relative position of the maximum tower in eta   (0-2).

  - For each group of 3 non-overlapping neighbouring 9×3-tower maxima objects in eta, set the veto flag if any of the following are true:

    - The maxima in the central region is not the central tower

    - The maxima in either the left or right regions is greater than the maxima in the central region

    - The maxima in either the left or right regions is equal to the the maxima in the central region and the maxima lies within the “<=” region as shown in Figure 2.

Figure 2: Geometry of the Jet Overlap Criteria for the 9×3 regions. The numbers down the left hand side indicate relative tower position in phi.

Figure 3: Left: The exhaustive list of permutations of the relative energies of A, B and C, where A, B and C are defined in Figure 2, and which of A, B and C would represent the most stringent condition for comparison with the central tower. Right: the same list with duplicates removed for clarity.

MaxPhi is a function which identifies which strip has its maximum furthest from the diagonal in the positive phi direction.

## Notes

The extreme lengths gone to within this algorithm are necessary to prevent an 80-way fan-in to a single point at every site and to then do 80-non reusable comparisons. As it stands each 3×1 comparison is reused three times, each 9×1 comparison reused three times and each 9x3 comparison reused 3 times and the biggest fan-in is only ever 3:1.

Algorithm: Donut pile-up estimation (DEPRECATED)

Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/PileUpEstimator.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/JetPipe.vhd |
| Internal entities | projects/calol2/algorithm_components/firmware/hdl/jets/JetBitonicSort.vhd |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd |

Generics

| Pipe lengths | strip9x1PipeInLength strip1x9PipeInLength pileUpPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | - |

I/O

| Inputs | Name | strip9x1PipeIn |
| --- | --- | --- |
|  | Type | tJetsPipe( strip9x1PipeInLength-1 DOWNTO 0 ) |
|  |  | Strips 9 in phi by 1 in eta, organized by eta-half/phi |
|  | Name | strip1x9PipeIn |
|  | Type | tJetsPipe( strip1x9PipeInLength-1 DOWNTO 0 ) |
|  | Description | Strips 1 in phi by 9 in eta, organized by eta-half/phi |
| Outputs | Name | pileUpPipeOut |
|  | Type | tJetsPipe( pileUpPipeOutLength-1 DOWNTO 0 ) |
|  | Description | An energy sum representing the estimated pileup in a region of size 9 in phi by 9 in eta, organized by eta-half/phi |

Latency

| Clock cycles | 5 |
| --- | --- |
| BX | 5/6 |

Description

- GENERATE loop over all towers in phi and both halves in eta and for each site

  - Select the four strips which abut the Jet on each side

  - Bitonic-sort the four strips by energy (see section “Algorithm: Jet Sorting”)

  - Add the middle two strips in the sorted list and left-shift by 2 (Multiply by 4) to compensate for relative area of strips and jet (should really be 4.5).

Notes

Since the Bitonic sort acts on “FilteredJet” objects, rather than simply the “Jet” objects, we must use the conversion function “ToFilteredJet” and define the local-type “tFilteredJetsInputsInPhi” to hold a 4-entry input for the Bitonic sort.

# Algorithm: Donut pile-up estimation

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/PileUpEstimator.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/JetPipe.vhd |
| Internal entities | projects/calol2/algorithm_components/firmware/hdl/jets/JetBitonicSort.vhd |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd |

## Generics

| Pipe lengths | jetVetoPipeInLength strip9x1PipeInLength strip1x9PipeInLength pileUpPipeOutLength |
| --- | --- |
| Pipe Offsets | stripPipeOffset |
| Other | - |

## I/O

| Inputs | Name | strip9x1PipeIn |
| --- | --- | --- |
|  | Type | tJetsPipe( strip9x1PipeInLength-1 DOWNTO 0 ) |
|  | Description | Strips 9 in phi by 1 in eta, organized by eta-half/phi |
|  | Name | strip1x9PipeIn |
|  | Type | tJetsPipe( strip1x9PipeInLength-1 DOWNTO 0 ) |
|  | Description | Strips 1 in phi by 9 in eta, organized by eta-half/phi |
|  | Name | jetVetoPipeIn |
|  | Type | tComparisonsPipe( jetVetoPipeInLength-1 DOWNTO 0 ) |
|  | Description | Boolean specifying whether the central tower of the region 9 in phi by 9 in eta has been vetoed by another tower in the region, organized by eta-half/phi |
| Outputs | Name | pileUpPipeOut |
|  | Type | tJetsPipe( pileUpPipeOutLength-1 DOWNTO 0 ) |
|  | Description | An energy sum representing the estimated pileup in a region of size 9 in phi by 9 in eta, organized by eta-half/phi |

## Latency

| Clock cycles | ? |
| --- | --- |
| BX | ? |

## Description

- GENERATE loop over all towers in phi and both halves in eta and for each site

  - Select the four strips which abut the Jet on each side

  - Apply the jet veto filter to calculate the donut only for jets which are not vetoed

  - Bitonic-sort the four strips by energy (see section “Algorithm: Jet Sorting”)

  - Add the middle two strips in the sorted list and left-shift by 2 (Multiply by 4) to compensate for relative area of strips and jet (should really be 4.5).

## Notes

Since the Bitonic sort acts on “FilteredJet” objects, rather than simply the “Jet” objects, we must use the conversion function “ToFilteredJet” and define the local-type “tFilteredJetsInputsInPhi” to hold a 4-entry input for the Bitonic sort.

By applying the veto before donut creation, we reduce the number of 4-input sorts by a factor of 4. The now-deprecated donut pile-up estimation accounted for 25% of the algorithm resource usage, purely because of the sorts.

Since we must now wait for the veto to arrive before we can do the sort + sum, this has had the effect of delaying the pile-up estimation by 3 clock-cycles compared to the now-deprecated donut pile-up estimation which calculated the estimates prior to the formation of the veto.

# Algorithm: Jet Overlap Filter

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/JetOverlapFilter.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/JetPipe.vhd projects/calol2/algorithm_components/firmware/hdl/jets/FilteredJetPipe.vhd projects/calol2/algorithm_components/firmware/hdl/jets/JetComparisonPipe.vhd |
| Internal entities | - |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd |

## Generics

| Pipe lengths | jetSumPipeInLength jetVetoPipeInLength filteredJetsPipeOutLength |
| --- | --- |
| Pipe Offsets | SumPipeOffset VetoPipeOffset |
| Other | - |

## I/O

| Inputs | Name | jetSumPipeIn |
| --- | --- | --- |
|  | Type | tJetsPipe( jetSumPipeInLength-1 DOWNTO 0 ) |
|  | Description | Energy sums 9 in phi by 9 in eta, organized by eta-half/phi |
|  | Name | jetVetoPipeIn |
|  | Type | tComparisonsPipe( jetVetoPipeInLength-1 DOWNTO 0 ) |
|  | Description | Boolean specifying whether the central tower of the region 9 in phi by 9 in eta has been vetoed by another tower in the region, organized by eta-half/phi |
| Outputs | Name | filteredJetsPipeOut |
|  | Type | tFilteredJetsPipe( filteredJetsPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Energy sums 9 in phi by 9 in eta + global coordinate, central tower guaranteed to be the highest energy, organized by eta-half/phi |

## Latency

| Clock cycles | 2 |
| --- | --- |
| BX | 1/3 |

## Description

- GENERATE loop over 1/4 towers in phi and both halves in eta and for each site

  - Select the jet for output based on which if any of the four overlapping jet sites was not vetoed.

  - Assign the value of phi to the jet for output based on the GENERATE parameter

## Notes

For the now-deprecated pile-up estimation, this same code is used to select both the 9×9 energy sum and the pile-up subtraction estimate based on the jet veto bit so that the pile-up subtraction is only done on the filtered jets, rather than on the 3/4 jets which will be rejected by the overlap filtering anyway. In the current pile-up estimation scheme, the veto is applied explicitly within the pile-up estimation step, before the estimate is made. For a global pile-up estimation, this step would again be required.

The overlap filtering must assign an explicit phi-value to the candidates or else their position will be lost. This could be a relative position (0-3), but since the candidates will go into an 18-way sort after this, I assign global phi (0-71).

# Algorithm: Pile-up subtraction

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/PileUpSubtraction.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/FilteredJetPipe.vhd |
| Internal entities | - |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd |

## Generics

| Pipe lengths | filteredJetsPipeInLength filteredPileUpPipeInLength pileUpSubtractedJetsPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | - |

## I/O

| Inputs | Name | filteredJetsPipeIn |
| --- | --- | --- |
|  | Type | tFilteredJetsPipe( filteredJetsPipeInLength-1 DOWNTO 0 ) |
|  | Description | Energy sums 9 in phi by 9 in eta + global phi pos., central tower guaranteed to be the highest energy, organized by eta-half/phi |
|  | Name | filteredPileUpPipeIn |
|  | Type | tFilteredJetsPipe( filteredPileUpPipeInLength-1 DOWNTO 0 ) |
|  | Description | An energy sum representing the estimated pileup in a region of size 9 in phi by 9 in eta, organized by eta-half/phi |
| Outputs | Name | pileUpSubtractedJetsPipeOut |
|  | Type | tFilteredJetsPipe( pileUpSubtractedJetsPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Corrected energy sums 9 in phi by 9 in eta + global coordinate, central tower guaranteed to be the highest energy, organized by eta-half/phi |

## Latency

| Clock cycles | 1 |
| --- | --- |
| BX | 1/6 |

## Description

- GENERATE loop over 1/4 towers in phi and both halves in eta and for each site

  - Convert both the Jet Sum energy and the Pileup estimate to integer form

  - Subtract the Pileup estimate from the Jet Sum energy

  - If the result is less than zero, set result to zero

  - If the result is greater than or equal to zero, output the result as an unsigned

## Notes

No offsets are provided for the two input types, since there is freedom in the preceding algorithms to adjust the timings such that both inputs become valid simultaneously

# Algorithm: MHT coefficients

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/rings/MHTcoefficients.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/rings/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/JetComparisonPipe.vhd projects/calol2/algorithm_components/firmware/hdl/rings/MHTcoefficientsPipe.vhd |
| Internal entities | - |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/rings/functions_pkg.vhd |

## Generics

| Pipe lengths | jetVetoPipeInLength MHTcoefficientsPipeOutLength |
| --- | --- |
| Pipe Offsets | jetVetoPipeOffset |
| Other | - |

## I/O

| Inputs | Name | jetVetoPipeIn |
| --- | --- | --- |
|  | Type | tComparisonsPipe( jetVetoPipeInLength-1 DOWNTO 0 ) |
|  | Description | Boolean specifying whether the central tower of the region 9 in phi by 9 in eta has been vetoed by another tower in the region, organized by eta-half/phi |
| Outputs | Name | MHTcoefficientsPipeOut |
|  | Type | tMHTcoefficientsPipe( MHTcoefficientsPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Global trigonometric coefficients of the non-vetoed jet (if any), organized by eta-half/phi |

## Latency

| Clock cycles | 1 |
| --- | --- |
| BX | 1/6 |

## Description

- GENERATE loop over 1/4 towers in phi and both halves in eta and for each site

  - Compile-time calculate Sin(phi) and Cos(phi) from GENERATE parameter

  - Select the coefficient for output based on which, if any, of the four overlapping jet sites was not vetoed.

# Algorithm: HT/MHT sums

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/rings/HTandMHTsums.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/rings/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/rings/MHTcoefficientsPipe.vhd projects/calol2/algorithm_components/firmware/hdl/rings/RingPipe.vhd |
| Internal entities | projects/calol2/algorithm_components/firmware/cgn/multiplier16Ux10S.xco projects/calol2/algorithm_components/firmware/hdl/rings/RingSegmentSum.vhd |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/rings/functions_pkg.vhd |

## Generics

| Pipe lengths | pileUpSubtractedJetsPipeInLength MHTcoefficientsPipeInLength ringPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | vectorJetShift |

## I/O

| Inputs | Name | pileUpSubtractedJetsPipeIn |
| --- | --- | --- |
|  | Type | tFilteredJetsPipe( pileUpSubtractedJetsPipeInLength-1 DOWNTO 0 ) |
|  | Description | Corrected energy sums 9 in phi by 9 in eta + global coordinate, central tower guaranteed to be the highest energy, organized by eta-half/phi |
|  | Name | MHTcoefficientsPipeIn |
|  | Type | tMHTcoefficientsPipe( MHTcoefficientsPipeInLength-1 DOWNTO 0 ) |
|  | Description | Global trigonometric coefficients of the non-vetoed jet (if any), organized by eta-half/phi |
| Outputs | Name | ringPipeOut |
|  | Type | tRingSegmentsPipe2( ringPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Scalar and vector sum of jet energy, organized by eta-half |

## Latency

| Clock cycles | 4 |
| --- | --- |
| BX | 2/3 |

## Description

- GENERATE loop over 1/4 towers in phi and both halves in eta and for each site

  - Multiply jet energy by the trigonometric coefficients from the corresponding MHTcoefficient object to get x- and y-components of vector

  - Right-shift the x- and y-components of the vector sum by vectorJetShift bits and truncate the least-significant bits

  - Store as a 4x1 ring-segment object

- GENERATE loop over 1/12 towers in phi and both halves in eta and for each site

  - Sum three non-overlapping neighbouring 4x1 ring-segment objects in phi into a 12x1 ring-segment objects

- GENERATE loop over 1/36 towers in phi and both halves in eta and for each site

  - Sum three non-overlapping neighbouring 12x1 ring-segment objects in phi into a 36x1 ring-segment objects

- GENERATE loop over 1/72 towers in phi and both halves in eta and for each site

  - Sum two non-overlapping neighbouring 36x1 ring-segment objects in phi into a 72x1 ring-segment objects

# Algorithm: ET/MET sums

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/rings/ETandMETsums.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd projects/calol2/algorithm_components/firmware/hdl/rings/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/JetPipe.vhd projects/calol2/algorithm_components/firmware/hdl/rings/RingPipe.vhd |
| Internal entities | projects/calol2/algorithm_components/firmware/hdl/rings/RingSegmentSum.vhd |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/rings/functions_pkg.vhd |

## Generics

| Pipe lengths | stripPipeInLength vectorStripPipeInLength ringPipeOutLength |
| --- | --- |
| Pipe Offsets | stripOffset vectorStripOffset |
| Other | - |

## I/O

| Inputs | Name | strips9x1PipeIn |
| --- | --- | --- |
|  | Type | tJetsPipe( stripPipeInLength-1 DOWNTO 0 ) |
|  | Description | Strips 9 in phi by 1 in eta, organized by eta-half/phi |
|  | Name | vectorStrips9x1PipeIn |
|  | Type | tRingSegmentsPipe( vectorStripPipeInLength-1 DOWNTO 0 ) |
|  | Description | Ring-segments 9 in phi by 1 in eta, organized by eta-half/phi |
| Outputs | Name | ringPipeOut |
|  | Type | tRingSegmentsPipe2( ringPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Scalar and vector sum of tower energy, organized by eta-half |

## Latency

| Clock cycles | 7 |
| --- | --- |
| BX | 1 1/6 |

## Description

- GENERATE loop over 1/9 towers in phi and both halves in eta and for each site

  - Map the total energy of a region of  9x1 towers in phi from the 9x1 strip object, and the components and towers over threshold from the 9x1 vector-strip objects into a single container object

- GENERATE loop over 1/18 towers in phi and both halves in eta and for each site

  - Sum two non-overlapping neighbouring 9x1 ring-segment objects in phi into an 18x1 ring-segment objects

- GENERATE loop over 1/36 towers in phi and both halves in eta and for each site

  - Sum two non-overlapping neighbouring 18x1 ring-segment objects in phi into a 36x1 ring-segment objects

- GENERATE loop over 1/72 towers in phi and both halves in eta and for each site

  - Sum two non-overlapping neighbouring 36x1 ring-segment objects in phi into a 72x1 ring-segment objects

# Algorithm: HT/MHT and ET/MET accumulation

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/rings/RingAccumulators.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/rings/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/rings/RingPipe.vhd |
| Internal entities | - |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/rings/functions_pkg.vhd |

## Generics

| Pipe lengths | ringPipeInLength accumulatedRingPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | scalarSumShift vectorSumShift |

## I/O

| Inputs | Name | ringPipeIn |
| --- | --- | --- |
|  | Type | tRingSegmentsPipe2( ringPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Scalar and vector sum of tower energy, organized by eta-half |
| Outputs | Name | accumulatedRingPipeOut |
|  | Type | tRingSegmentsPipe2( ringPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Scalar and vector sum of tower energy, organized by eta-half, accumulated over the half-barrel |

## Latency

| Clock cycles | 1 |
| --- | --- |
| BX | 1/6 |

## Description

- Reset the accumulation if the incoming data is not valid

- The scalar sum of the incoming ring segment is right-shifted by scalarSumShift bits and the least-significant bits truncated

- The vector-sum components of the incoming ring segment are right-shifted by vectorSumShift bits and the least-significant bits truncated

- Otherwise, if the data is valid add it to existing accumulation and mark as valid

## Notes

The truncations make sure that the accumulated sum doesn’t overflow the 20-bit signed-type used to store the value and bring us back into the realm of sanity.

# Algorithm: Jet Sorting

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/JetBitonicSort.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/FilteredJetPipe.vhd |
| Internal entities | - |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd projects/calol2/algorithm_components/firmware/hdl/common/functions_pkg.vhd |

## Generics

| Pipe lengths | filteredJetsPipeInLength sortedJetsPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | - |

## I/O

| Inputs | Name | filteredJetsPipeIn |
| --- | --- | --- |
|  | Type | tFilteredJetsPipe( filteredJetsPipeInLength-1 DOWNTO 0 ) |
|  | Description | Corrected energy sums 9 in phi by 9 in eta + global coordinate, central tower guaranteed to be the highest energy, organized by eta-half/phi |
| Outputs | Name | sortedJetsPipeOut |
|  | Type | tFilteredJetsPipe( sortedJetsPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Corrected energy sums 9 in phi by 9 in eta + global coordinate, central tower guaranteed to be the highest energy, organized by eta-half/sort-index |

## Latency

| Clock cycles | 14 |
| --- | --- |
| BX | 2 1/3 |

## Description

- A fully-recursive implementation of a truncating, any-size bitonic sort of jet candidates. For more information on bitonic sorting algorithms, see

  - http://en.wikipedia.org/wiki/Bitonic_sorter

  - http://www.iti.fh-flensburg.de/lang/algorithmen/sortieren/bitonic/oddn.htm

## Notes

VHDL (or at least the ISE VHDL parser) does not like recursive constructs; they have to be carefully constructed to work at all.

In particular “xxx : entity work.yyy” type instantiations do not work, nor do COMPONENT declarations at global scope: COMPONENT declarations must be made in the ARCHITECTURE preamble.

I have modified the standard bitonic sort so that an output size is specified. Once an entry is guaranteed to fail to make it into the output list, it is discarded so as to save resources and latency. Setting the output size to the input size keeps all candidates.

A non-power-of-two bitonic sort has different sized sorts on each branch. Different sized sorts have different latencies, and so each branch must be latency compensated. The latency compensation is automatically calculated at compile-time in this implementation, the latency itself being calculated by a recursive function call on the GENERIC parameters.

# Algorithm: Accumulating Jet Sorting

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/jets/JetBitonicSort.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/jets/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/jets/FilteredJetPipe.vhd |
| Internal entities | - |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/jets/functions_pkg.vhd projects/calol2/algorithm_components/firmware/hdl/common/functions_pkg.vhd |

## Generics

| Pipe lengths | sortedJetsPipeInLength accumulatedSortedJetsPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | - |

## I/O

| Inputs | Name | sortedJetsPipeIn |
| --- | --- | --- |
|  | Type | tFilteredJetsPipe( filteredJetsPipeInLength-1 DOWNTO 0 ) |
|  | Description | Corrected energy sums 9 in phi by 9 in eta + global coordinate, central tower guaranteed to be the highest energy, organized by eta-half/sort-index |
| Outputs | Name | accumulatedSortedJetsPipeOut |
|  | Type | tFilteredJetsPipe( sortedJetsPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Corrected energy sums 9 in phi by 9 in eta + global coordinate, central tower guaranteed to be the highest energy, organized by eta-half/sort-index, accumulated over the half-barrel |

## Latency

| Clock cycles | 18 |
| --- | --- |
| BX | 3 |

## Description

- An accumulating sort which takes a sorted list and returns a sorted list integrated over time.

- The generic bitonic sort functionality is reused within this module.

- Because sorting has a finite latency, accumulation is complicated. The implementation used here is an interleaved accumulating sort, where the outputs are fed into a pipeline and the interleaved sorts then merged into a final sorted list. This is shown in Figure 4.

Figure 4 : The architecture of the accumulating bitonic sort showing the mechanism used to compensate for the latency of merging two sorted lists

## Notes

VHDL (or at least the ISE VHDL parser) does not like recursive constructs; they have to be carefully constructed to work at all.

In particular “xxx : entity work.yyy” type instantiations do not work, nor do COMPONENT declarations at global scope: COMPONENT declarations must be made in the ARCHITECTURE preamble.

I have modified the standard bitonic sort so that an output size is specified. Once an entry is guaranteed to fail to make it into the output list, it is discarded so as to save resources and latency. Setting the output size to the input size keeps all candidates.

A non-power-of-two bitonic sort has different sized sorts on each branch. Different sized sorts have different latencies, and so each branch must be latency compensated. The latency compensation is automatically calculated at compile-time in this implementation, the latency itself being calculated by a recursive function call on the GENERIC parameters.

The period of interleaving and thus size of the final sort depends on the latency of the initial sort, which in-turn depends on the size of the initial sort. All sizes and latencies are automatically calculated and invisible to the user.

# Algorithm: Coordinate conversion

## Files

| Top-level | projects/calol2/algorithm_components/firmware/hdl/rings/CoordinateConversion.vhd |
| --- | --- |
| Types | projects/calol2/algorithm_components/firmware/hdl/rings/types_pkg.vhd |
| Pipes | projects/calol2/algorithm_components/firmware/hdl/rings/RingPipe.vhd projects/calol2/algorithm_components/firmware/hdl/rings/PolarRingPipe.vhd |
| Internal entities | - |
| Utilities | projects/calol2/algorithm_components/firmware/hdl/rings/functions_pkg.vhd |

## Generics

| Pipe lengths | ringPipeInLength polarRingPipeOutLength |
| --- | --- |
| Pipe Offsets | - |
| Other | - |

## I/O

| Inputs | Name | ringsPipeIn |
| --- | --- | --- |
|  | Type | tFilteredRingsPipe( ringsPipeInLength-1 DOWNTO 0 ) |
|  | Description | Scalar and vector sum of tower energy, organized by eta-half, accumulated over the half-barrel |
| Outputs | Name | polarRingPipeOut |
|  | Type | tPolarRingSegmentPipe( polarRingPipeOutLength-1 DOWNTO 0 ) |
|  | Description | Scalar and vector sum of tower energy, organized by eta-half, accumulated over the half-barrel, in polar coordinates |

## Latency

| Clock cycles | ? |
| --- | --- |
| BX | ? |

## Description

- An initial angle and a sign bit are set according to the quadrant in which the Cartesian vector components lie. The sign bit specifies the direction of the rotation from the initial angle.

- The magnitude of the vector components is taken so that the coordinate rotation is performed in the primary quadrant.

- Eight, pipelined, coordinate-rotation steps are applied, each updating the size of the Cartesian components and the angle. The y-component of the Cartesian coordinates is used as the control-variable and the x-component is, therefore, the component being updated for the magnitude. The constants needed for the rotation are calculated at compile-time.

- The magnitude (x-component) is multiplied by a constant value (which depends on the number of rotation steps and a scaling factor) and then right-shifted by the scaling factor. The value of the constant is calculated at compile-time.

## Notes

This CORDIC algorithm is based on python code (albeit modified to handle the four-quadrant input) available here:

http://code.activestate.com/recipes/576792-polar-to-rectangular-conversions-using-cordic/

CORDICs are magic:

http://en.wikipedia.org/wiki/CORDIC